Task: Sierra Leone Flood Exposure Screening - Schools in Waterway Risk Corridors

Flood exposure around educational facilities poses a direct risk to student safety, school accessibility, and continuity of learning, particularly during seasonal rainfall events in Sierra Leone. This analysis aims to identify which schools fall within a defined flood-risk corridor derived from the national waterway network, and to separate vulnerable facilities from those located in safer areas. The outputs will support disaster preparedness, education infrastructure planning, and resilience-focused investment by providing a clear spatial assessment of schools potentially exposed to flood-related disruption and environmental risk.

You will be working with two layers: water_lines.gpkg and edu_facilities.gpkg as the primary analysis inputs, and SLE_adm2.gpkg included solely to provide administrative boundary context for the map. Both layers must be in EPSG:32628 (WGS 84 / UTM Zone 28N) before any processing begins. You must verify and reproject, if necessary (use the reprojected layers for all analysis), as all area and length calculations depend on metric units.

The core analytical operation involves generating a 200-metre fixed-distance buffer around all waterway features, using the Buffer tool under Vector Geoprocessing Tools, setting the distance to 200 meters, and ensuring the "Dissolve result" option is checked so that overlapping buffer zones along the line network merge into a single unified polygon. This dissolved buffer represents the flood-risk corridor, the zone of direct exposure to waterway flooding, and is saved as waterway_buffer_200m.gpkg.

Two separate location-based extractions ("Extract by Location" operations) must be performed on this resulting buffer layer against the edu_facilities.gpkg layer, one for each risk class. For flood-prone schools, the geometric predicate is intersects: this captures any school point that lies inside, touches the edge of, or overlaps with the dissolved buffer polygon, and saves the result as schools_flood_prone.gpkg. For safe schools, with the same tool but using the are disjoint predicate instead: this selects only school points that share no spatial relationship whatsoever with the buffer polygon, no overlap, no boundary contact, and saves that subset as schools_safe.gpkg. Together, the two operations partition the full edu_facilities.gpkg dataset into two mutually exclusive subsets that account for every school record. Both outputs must carry all original attribute fields from edu_facilities.gpkg without any addition, removal, or renaming at this stage.

Using the Field Calculator, a new string attribute field named flood_risk (length 20) is to be added to each subset independently, hard-coding the expression 'HIGH' for schools_flood_prone.gpkg and 'LOW' for schools_safe.gpkg. This produces two attribute-enriched temporary outputs that are then fed into the Merge Vector Layers tool, which stacks them into a single unified layer saved as edu_facilities_flood_risk.gpkg. The merge preserves all original fields from edu_facilities.gpkg alongside the new flood_risk column, giving every school record in the final layer a clearly assigned risk classification. This consolidated layer is the primary deliverable, and is styled on the  flood_risk field to assist visual interpretation using categorized symbols: flood-prone (HIGH) schools rendered in red, safe schools (LOW) in green, and the waterway buffer as a semi-transparent blue fill with a darker blue outline.

Deliver a project folder, Schools_Flood_Risk, containing the input layers (water_lines.gpkg, edu_facilities.gpkg, and SLE_adm2.gpkg), all the derived output layers (waterway_buffer_200m.gpkg, schools_flood_prone.gpkg, schools_safe.gpkg, and edu_facilities_flood_risk.gpkg,), and Flood_Exposure_Screening.qgz saved with a relative path, preserving all the expressions and layer stylings. The QGIS project reopens without broken paths, missing layers, or mismatched CRS.